208BA Wireless Camera & Receiver Set User Manual

Features

208BA – 120WA CMOS Black/White Wireless Micro Camera and Receiver set is an enhanced model with audio capability. B/W camera is known for its high sensitivity, high quality reliable picture, and low illumination requirement. It is used at homes, workshops, warehouses, schools, offices, stores, gas stations, and any places that wiring is not possible. It is also used by private investigators and law enforcement agencies for surveillance and video monitoring. Main features include wireless transmission and reception, long reception range, small compact size and minimal weight, low power consumption, high sensitivity, low maintenance, easy installation and operation, and easy concealment. Beside the standard lens that comes with the 208BA camera, a series of interchangeable lenses for different usages/purposes can be purchased to server your special needs.

Package Content

One (1) 208BA Wireless Micro Camera with standard lens, One (1) 120WA Wireless Receiver, One (1) DC 9V Power Adapter (for receiver), One (1) DC 8V Power Adapter (for camera), One (1) 9V Battery Clip, One (1) RCA type Audio/Video Cable.

Operation Instructions

(see Typical Connecting Model on the back for reference)

- Attach (twist) Reception Antenna to Wireless Receiver.

- Connect Wireless Receiver to Monitor with Video Cable.

- Connect DC 9V Power Adapter to Power Jack of Wireless Receiver.

- Connect DC 8V Power Adapter to Power Supply Socket of Wireless Camera. (Note incorrectly connecting the Wireless Camera to the 9V Power Adapter may cause permanent camera damage.)

- Remove (pop off) the lens-protecting cap from the camera lens. Now everything is connected. Adjust Frequency Controller on Wireless Receiver to a position that best picture quality is achieved.

- Adjust Wireless Camera lens to a position that picture is in focus.

- Mount Wireless Camera to a fixed position if needed.

Specifications

| Effective Reception Range | Up to 1000 feet “line of sight” |
| --- | --- |
| Resolution | 420 lines |
| Scan Frequency | EIA: 60 Hz |
| Minimum Illumination | 0.5 LUX |
| TV System | NTSC |
| Power Supply | DC 8V, 9V |
| Power Consumption | < 960 mW |
| Camera Dimensions | 25 mm x 33 mm x 33 mm |
| Receiver Dimensions | 59 mm x 115 m x 20 mm |

Usage Agreement

The manufacture or the distributor/reseller of this device(s) cannot be held liable under any circumstances if the device(s) is used for illegal purposes, offenders are subject to severe legal punishments by government law enforcement agencies.

208BA – 120WA Typical Connecting Model (not to scale)

Frequency Controller

Reception Antenna

Monitor (not included)

| DC 9V Adapter Wireless Receiver (120WA) Power Jack |  |  |
| --- | --- | --- |
| Transmission Antenna |  | A/V Cable |
| Wireless Camera (208BA) Battery Clip Power Supply Socket |  |  |
|  | or | DC 8V Adapter |
| # Lithium Rechargeable Battery (not included) |  | Interchangeable Lenses (not included) Available at www.RockyAmericas.com |